Below is a complete, ready-to-run plan for turning a Raspberry Pi 4 + ReSpeaker 2-Mic Pi HAT into a tiny “sound-server” that will play the various game-effects the ESP-32 sends over UDP.
I split the answer into three parts:

| Part | What it covers |
| --- | --- |
| A – Architecture & protocol | How the two boards talk, what data is sent, and why UDP is fine for this case. |
| B – ESP-32 side (sender) | Minimal changes to the sketch (or a separate helper file) so the ESP-32 emits the right UDP packets when a SoundEvent occurs. |
| C – Raspberry Pi side (receiver / player) | Wiring of the ReSpeaker, enabling I²S output on the Pi, installing the software stack, and a ready-made Python daemon that plays the sounds. |

## A- Architecture & UDP protocol

Wi-Fi (or Ethernet)          UDP (port 9999)

+--------------------------+   +--------------------------+

| ESP-32 (game logic)      |──►| Raspberry Pi (sound hub) |

+--------------------------+   +--------------------------+

ESP-32  →  short packet (1 byte)  = SoundEvent enum

Pi      →  plays the matching audio file (WAV/MP3) through the ReSpeaker DAC

### Why UDP works

| Reason |  |
| --- | --- |
| Latency – UDP has no hand-shaking, so the packet arrives in ≈ 1 ms (the only delay is the Wi-Fi round-trip). |  |
| Reliability is not critical – Missing a single shot-sound or start chime is harmless; the gameplay itself stays smooth. |  |
| Low bandwidth – each packet is 1 byte (or at most a few bytes for a tone-pair) → no congestion. |  |
| Stateless – the ESP-32 never waits for an ACK; it can keep its 60 FPS loop unaffected. |  |

### Simple packet format

| Byte | Meaning |
| --- | --- |
| 0x00 | EVT_NONE – (ignore) |
| 0x01 | EVT_START – play “game start” jingle |
| 0x02 | EVT_WIN – level-won fanfare |
| 0x03 | EVT_LOSE – game-over buzzer |
| 0x04 | EVT_MISTAKE – wrong-colour penalty |
| 0x05 | EVT_HIT_SUCCESS – successful enemy hit |
| 0x06 – 0x09 | EVT_SHOT_BLUE / RED / GREEN / WHITE – player shot sounds |

All other values are ignored.

If you ever need custom tones (frequency + duration) you can extend the packet to 3 bytes:

[EVENT_ID][freq_H][freq_L]    // freq = 16-bit value (0-65535) in Hz

[EVENT_ID][duration]         // duration = 1-byte, 10 ms steps (0-2550 ms)

But the simplest “one-byte” approach already covers the stock game.

## B- ESP-32 side – sending the UDP packet

You don’t have to re-enable the stripped audio code – just add a tiny UDP client that fires whenever the original playSound() or playShotSound() functions are called.

### 1- Add the UDP library & global objects

#include <WiFiUdp.h>

WiFiUDP udp;               // UDP socket

const char*  UDP_TARGET_IP   = "192.168.4.1";   // <-- IP of the Pi on the same LAN

const uint16_t UDP_TARGET_PORT = 9999;          // any free port you like

Set UDP_TARGET_IP to the static IP of the Pi (or use its hostname if you have mDNS).

### 2- Tiny helper that sends a single byte

void udpSendEvent(uint8_t eventId) {

udp.beginPacket(UDP_TARGET_IP, UDP_TARGET_PORT);

udp.write(&eventId, 1);

udp.endPacket();            // fire-and-forget – no ACK needed

}

### 3- Replace the stub playSound() and playShotSound()

void playSound(SoundEvent evt) {

//  Keep the existing Serial debug line if you want:

Serial.print(F("UDP SoundEvent = ")); Serial.println(evt);

udpSendEvent((uint8_t)evt);          // <-- send the enum value

}

void playShotSound(int color) {

// Map colour to its shot-event

SoundEvent ev = EVT_SHOT_BLUE;      // default

switch (color) {

case 1: ev = EVT_SHOT_BLUE;  break;

case 2: ev = EVT_SHOT_RED;   break;

case 3: ev = EVT_SHOT_GREEN;break;

case 7: ev = EVT_SHOT_WHITE;break;

}

Serial.print(F("UDP Shot = ")); Serial.println(ev);

udpSendEvent((uint8_t)ev);

}

That’s it – no more audioQueue or I²S code is needed.

### 4- Initialise the UDP socket once (e.g. in setup())

void setup() {

// … existing Wi-Fi connection code …

udp.begin(0);               // pick any local port (0 = random)

// … the rest of your setup …

}

Tip: If you ever want to verify that packets actually reach the Pi, you can run tcpdump -n -i any udp port 9999 on the Pi while the game is running.

## C- Raspberry Pi side – receiving UDP and playing through the ReSpeaker

Below are the exact steps to turn the Pi into a headless sound server that:

- listens on UDP-port 9999,

- maps the received byte to a local audio file,

- plays the file through the ReSpeaker (or the 3.5 mm jack if you prefer).

### 1 Wiring the ReSpeaker 2-Mic Pi HAT

| Pin on HAT | Connection on Pi 4 (GPIO) |
| --- | --- |
| 3.3 V | Pin 1 (3V3) |
| GND | Pin 6 (GND) |
| I²S WS | Pin 35 (GPIO 19) – LRCK |
| I²S CLK | Pin 12 (GPIO 18) – BCLK |
| I²S DIN | Pin 40 (GPIO 21) – SDIN (for microphones) |
| I²S DOUT | Pin 38 (GPIO 20) – SDOUT (audio out) |

The ReSpeaker’s WM8960 codec supports both capture and playback over the same I²S bus.
For our use-case we only need playback.

If you simply want to use the Pi’s built-in HDMI/3.5 mm audio you can skip the wiring entirely – the same software will work, just change the ALSA output device.

### 2- Enable I²S on the Pi

Edit /boot/config.txt (add at the end of the file, then reboot):

sudo nano /boot/config.txt

Add:

dtoverlay=i2s-mmap

dtoverlay=wm8960,i2c=on

dtparam=i2s=on

The wm8960 overlay configures the codec; the i2s-mmap overlay gives a simple PCM device (/dev/snd/...).

After saving, reboot:

sudo reboot

### 3- Verify the ALSA device

When the Pi comes back up, run:

aplay -l

You should see something like:

card 0: WM8960 [wm8960], device 0: WM8960 PCM [wm8960 PCM]

Subdevices: 1/1

Subdevice #0: subdevice #0

If you only want the HDMI or analog jack you’ll see card 0: ALSA etc. – just note the card number for later.

### 4- Install the required software

sudo apt update

sudo apt install -y python3 python3-pip ffmpeg   # ffmpeg for any type of audio file

pip3 install --user python-rtmidi    # not needed for simple playback but useful later

We’ll use aplay (ALSA command-line player) from a very lightweight Python daemon.

### 5- Prepare the sound library

Create a folder, e.g. /home/pi/sounds, and put a WAV or MP3 file for each event.
Name them exactly as the enum values so the mapping code can be trivial:

| File name (lower-case) | Content (example) |
| --- | --- |
| evt_start.wav | Intro jingle |
| evt_win.wav | Victory fanfare |
| evt_lose.wav | Game-over buzz |
| evt_mistake.wav | Penalty beep |
| evt_hit_success.wav | “Kill” pop |
| evt_shot_blue.wav | Blue laser |
| evt_shot_red.wav | Red laser |
| evt_shot_green.wav | Green laser |
| evt_shot_white.wav | White/combo laser |

Make sure they are 16-bit PCM, 44.1 kHz for best compatibility (ffmpeg can convert them):

ffmpeg -i source.mp3 -ar 44100 -ac 2 -sample_fmt s16 evt_start.wav

### 6- Python UDP-listener daemon

Create /home/pi/sound_server.py :

#!/usr/bin/env python3

import socket

import subprocess

import os

import sys

import signal

# -------------------------------------------------------------

# Configuration

# -------------------------------------------------------------

UDP_IP   = "0.0.0.0"          # listen on all interfaces

UDP_PORT = 9999

SOUND_DIR = "/home/pi/sounds"

# Map enum byte → filename (lowercase, without extension)

EVENT_MAP = {

0x01: "evt_start.wav",

0x02: "evt_win.wav",

0x03: "evt_lose.wav",

0x04: "evt_mistake.wav",

0x05: "evt_hit_success.wav",

0x06: "evt_shot_blue.wav",

0x07: "evt_shot_red.wav",

0x08: "evt_shot_green.wav",

0x09: "evt_shot_white.wav",

}

# -------------------------------------------------------------

# Helper: play an audio file via ALSA (aplay)

# -------------------------------------------------------------

def play_sound(fname):

full_path = os.path.join(SOUND_DIR, fname)

if not os.path.isfile(full_path):

print(f"[WARN] missing {full_path}")

return

# Using aplay is the simplest, non-blocking approach:

# '-q' = quiet, '-D plughw:0,0' = first PCM device (adjust if needed)

subprocess.Popen(["aplay", "-q", full_path])

# -------------------------------------------------------------

# Main UDP loop

# -------------------------------------------------------------

def main():

# Create UDP socket

sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

sock.bind((UDP_IP, UDP_PORT))

print(f"[INFO] Listening for UDP packets on {UDP_IP}:{UDP_PORT}")

# Graceful termination handling

def handler(sig, frame):

print("\n[INFO] Shutting down.")

sock.close()

sys.exit(0)

signal.signal(signal.SIGINT, handler)

signal.signal(signal.SIGTERM, handler)

while True:

try:

data, addr = sock.recvfrom(32)           # max 32 bytes (we only need 1)

if len(data) == 0:

continue

ev = data[0]

fname = EVENT_MAP.get(ev)

if fname:

print(f"[EVENT] {ev:#02x} from {addr} -> {fname}")

play_sound(fname)

else:

print(f"[UNKNOWN] byte {ev:#02x} from {addr}")

except Exception as e:

print("[ERR] ", e)

if __name__ == "__main__":

main()

Make it executable:

chmod +x /home/pi/sound_server.py

### 7- Run it as a systemd service (auto-start on boot)

Create /etc/systemd/system/rgb_invaders_sound.service :

[Unit]

Description=RGB Invaders UDP Sound Server

After=network.target

[Service]

User=pi

WorkingDirectory=/home/pi

ExecStart=/usr/bin/python3 /home/pi/sound_server.py

Restart=always

RestartSec=2

[Install]

WantedBy=multi-user.target

Enable & start:

sudo systemctl daemon-reload

sudo systemctl enable rgb_invaders_sound.service

sudo systemctl start rgb_invaders_sound.service

Check the logs:

journalctl -u rgb_invaders_sound -f

You should see lines such as:

[INFO] Listening for UDP packets on 0.0.0.0:9999

[EVENT] 0x01 from ('192.168.4.23', 56884) -> evt_start.wav

When you press a button on the ESP-32 the corresponding sound will play instantly through the ReSpeaker (or any ALSA output you chose).

## Summary of the whole workflow

| Step | Action | Result |
| --- | --- | --- |
| 1. | Wire the ReSpeaker HAT to the Pi’s 3.3 V, GND, and I²S pins (GPIO 18/19/20). | Pi can output PCM audio via the WM8960 codec. |
| 2. | Enable the WM8960 overlay in /boot/config.txt and reboot. | An ALSA PCM device (hw:0,0) appears. |
| 3. | Put WAV/MP3 files named evt_*.wav into /home/pi/sounds. | Sound library ready. |
| 4. | Run the Python daemon (systemd service). | Pi constantly listens on UDP 9999 and plays any event it receives. |
| 5. | Add the tiny UDP client (udpSendEvent) to the ESP-32 sketch and replace playSound()/playShotSound(). | The ESP-32 now sends 1-byte packets every time the game wants a sound. |
| 6. | Power both boards on the same LAN (or connect the Pi to the ESP-32’s AP). | Press a button → immediate audio feedback on the ReSpeaker. |

## Optional Enhancements

| Idea | How-to-Implement |
| --- | --- |
| Volume control from the UI | Add a /setvolume HTTP endpoint that writes a new value to a text file; modify sound_server.py to read it and call amixer set Master <value>% before each playback. |
| Dynamic melodies (not just static WAV files) | Send a second UDP packet that carries freq,dur pairs (e.g., "523,80;659,80"). On the Pi parse the string and synthesize on-the-fly with numpy + scipy or simpleaudio. |
| Checksum / reliability | Prefix each packet with a magic byte (e.g., 0xAA) and ignore any packet that doesn’t start with it; you still keep the low-latency nature. |
| Multiple sound-servers | Broadcast to a multicast address (239.0.0.10:9999) so you can have more than one Pi playing the same effect (great for larger installations). |
| Sync the LED strip with the Pi | Send a second UDP stream with a timestamp (millis()) so the Pi can light a small external LED or a visualizer in perfect step with the game. |

### TL;DR (cheat-sheet)

- ESP-32 – add a WiFiUDP object, call udpSendEvent((uint8_t)event) inside playSound()/playShotSound().

- Raspberry Pi – enable the WM8960 I²S overlay, install aplay/ffmpeg, put the 9 WAV files in /home/pi/sounds, run the provided Python daemon listening on UDP 9999 and playing the mapped file via aplay.

- Wire → ReSpeaker HAT → I²S pins (GPIO 18, 19, 20).

- Result – every in-game event triggers a crisp, low-latency sound on the Pi without touching the ESP-32’s CPU or memory budget.

You’re now ready to give your RGB Invaders a full-blown audio soundtrack using a Raspberry Pi 4 and the ReSpeaker HAT. Happy hacking – and may your LEDs flash in perfect rhythm with those freshly-added sound effects! 🎶🚀