Below is a complete wiring guide and a minimal Arduino-style test sketch that lets you control 8 × tri-colour (common-cathode) LEDs with two MCP23017 I²C port-expanders on an ESP-32.
The code uses the Adafruit MCP23017 library, sets each pin as an output, and then cycles each colour of each LED one-by-one so you can verify that the hardware and software are working.

## 1️⃣ What you need (parts)

| Qty | Part |
| --- | --- |
| 2 | MCP23017 breakout board (or plain IC on a bread-board) |
| 8 | Common-cathode RGB LEDs (3 pins + common cathode) |
| 24 | 330 Ω – 470 Ω current-limiting resistors (one per colour pin) |
| 1 | 3.3 V power supply (the ESP-32’s 3.3 V rail) |
| 1 | Bread-board / perfboard and jumpers |
| 1 | ESP-32 development board (e.g. ESP-32 DevKit V1) |
| 1 | Libraries (installed through the Arduino Library Manager): • Wire (built-in) • Adafruit MCP23017 (Adafruit_MCP23017) |

## 2️⃣ Pin-assignment – how the 24 LED pins are mapped

+3.3 V (common anode of each RGB LED)

|

┌───────────────────────────────────────┐

│   MCP23017 #1  (addr = 0x20)          │

│   Port A (GPIOA0-A7)  →  Red   of LED0-7   │

│   Port B (GPIOB0-B7)  →  Green of LED0-7   │

└───────────────────────────────────────┘

I²C SDA → ESP-32 GPIO21

I²C SCL → ESP-32 GPIO22

VDD  → 3.3 V

GND  → GND

A0-A2 → GND (address 0x20)

┌───────────────────────────────────────┐

│   MCP23017 #2  (addr = 0x21)          │

│   Port A (GPIOA0-A7)  →  Blue  of LED0-7   │

│   (Port B not used – free for future)      │

└───────────────────────────────────────┘

A0 → VCC (to make address 0x21)

A1 → GND

A2 → GND

### Wiring each LED

- Common cathode of the LED → GND of the system.

- For each colour (R, G, B) of each LED:

- MCP pin (output) ──> 330 Ω resistor ──> LED colour pin ──> +3.3 V

The resistor limits the current to roughly 10-15 mA (see calculation below).

#### Resistor value calculation (3.3 V supply)

VCC = 3.30 V

Vf (typical) = 2.0 V (red) / 3.0 V (green) / 3.2 V (blue)

Desired I  = 12 mA

R = (VCC – Vf) / I

≈ (3.3 – 2.0) / 0.012 ≈ 108 Ω  → round up to 220 Ω (safer)

≈ (3.3 – 3.0) / 0.012 ≈ 25 Ω   → round up to 120 Ω (but 330 Ω is fine for all colours)

330 Ω works for all three colours and gives ≈ 5-10 mA, well below the MCP’s 25 mA per-pin absolute limit.
Even if you light all 24 colour pins at once you’d draw about 200 mA, which is above the MCP’s total-package limit (≈ 70 mA).
Never turn every colour on simultaneously – the test sketch only lights one colour at a time, so the current stays < 10 mA × 8 ≈ 80 mA per chip, still safe because each chip only drives one colour (8 pins) at a time.

## 3️⃣ Wiring diagram (text / ASCII)

ESP-32                           MCP23017 #1 (addr 0x20)               MCP23017 #2 (addr 0x21)

3.3V ───+----------------------+ VDD                               VDD

GND  ───+----------------------+ GND                               GND

GPIO21 (SDA) ────────────────┬─── SDA                              SDA (shared)

GPIO22 (SCL) ────────────────┴─── SCL                              SCL (shared)

A0 ── GND (addr 0x20)                              A0 ── VCC (addr 0x21)

A1 ── GND                                         A1 ── GND

A2 ── GND                                         A2 ── GND

MCP #1 Port A (GPIOA0-A7) ──► 330 Ω ──► Red leg of LED0…LED7

MCP #1 Port B (GPIOB0-B7) ──► 330 Ω ──► Green leg of LED0…LED7

MCP #2 Port A (GPIOA0-A7) ──► 330 Ω ──► Blue leg of LED0…LED7

(Port B of MCP #2 unused – free for future)

All LED colour pins share a common +3.3 V rail.

All LED cathodes (common) are tied to GND.

Tip: Keep the wiring short and use a bread-board or a proto-shield so the 8 × 3 = 24 resistors are neatly organized in rows.

## 4️⃣ Test sketch (Arduino-style)

/* --------------------------------------------------------------

TEST OF TWO MCP23017s DRIVING 8 COMMON-CATHODE RGB LEDs

-------------------------------------------------------------- */

#include <Wire.h>

#include <Adafruit_MCP23017.h>

/* --------------------------------------------------------------

I²C address configuration

-------------------------------------------------------------- */

// MCP23017 #1 – pins A0-A2 tied to GND  => address 0x20

// MCP23017 #2 – A0 tied to VCC, A1-A2 to GND => address 0x21

#define MCP1_ADDR 0x20

#define MCP2_ADDR 0x21

/* --------------------------------------------------------------

Create the two MCP objects

-------------------------------------------------------------- */

Adafruit_MCP23017 mcpRedGreen;   // handles RED (A) and GREEN (B) pins

Adafruit_MCP23017 mcpBlue;       // handles BLUE (A) pins

/* --------------------------------------------------------------

Helper: turn a single colour of a given LED on or off

-------------------------------------------------------------- */

// ledIndex : 0 … 7

// colour   : 'R', 'G', or 'B'

// state    : true = LED on, false = LED off

void setLED(uint8_t ledIndex, char colour, bool state) {

if (ledIndex > 7) return;   // safety

// MCP23017 pins are active-HIGH when configured as outputs.

// For a common-cathode LED we *sink* current:  state==true → write LOW.

// (If you wire the LEDs as common-anode, invert the logic below.)

uint8_t pin = ledIndex;               // same number on each port

// LOW = sink current → LED ON

uint8_t value = state ? LOW : HIGH;

switch (colour) {

case 'R':                 // RED pins are on Port A of MCP #1

mcpRedGreen.digitalWrite(pin, value);

break;

case 'G':                 // GREEN pins are on Port B of MCP #1

mcpRedGreen.digitalWrite(pin + 8, value);   // Port B = pins 8-15

break;

case 'B':                 // BLUE pins are on Port A of MCP #2

mcpBlue.digitalWrite(pin, value);

break;

}

}

/* --------------------------------------------------------------

Setup – initialise I²C, configure pins as outputs, turn everything off

-------------------------------------------------------------- */

void setup() {

Serial.begin(115200);

Serial.println(F("\n=== MCP23017 RGB-LED demo ==="));

// ----- I²C -----

Wire.begin(21, 22);                 // SDA = GPIO21, SCL = GPIO22

// ----- Initialise both expanders -----

mcpRedGreen.begin(MCP1_ADDR);        // address 0x20

mcpBlue.begin(MCP2_ADDR);            // address 0x21

// ----- Set every pin to OUTPUT and turn them OFF (HIGH = off for common-cathode) -----

for (uint8_t i = 0; i < 16; i++) {

mcpRedGreen.pinMode(i, OUTPUT);

mcpRedGreen.digitalWrite(i, HIGH);   // HIGH = LED off (no sinking)

}

for (uint8_t i = 0; i < 8; i++) {

mcpBlue.pinMode(i, OUTPUT);

mcpBlue.digitalWrite(i, HIGH);

}

Serial.println(F("All LEDs initialised – starting demo..."));

}

/* --------------------------------------------------------------

Loop – simple demo: walk through the 8 LEDs, light each colour in turn

-------------------------------------------------------------- */

void loop() {

const uint16_t delayMs = 200;   // how long each colour stays lit

for (uint8_t led = 0; led < 8; led++) {

// ---- RED ----

setLED(led, 'R', true);

delay(delayMs);

setLED(led, 'R', false);

// ---- GREEN ----

setLED(led, 'G', true);

delay(delayMs);

setLED(led, 'G', false);

// ---- BLUE ----

setLED(led, 'B', true);

delay(delayMs);

setLED(led, 'B', false);

}

// Optional: pause a second before the next sweep

delay(1000);

}

### How the sketch works

| Step | What happens |
| --- | --- |
| setup() – Initialises the I²C bus on GPIO 21/22. – Creates two MCP objects at addresses 0x20 and 0x21. – Configures all 24 pins as OUTPUT and sets them HIGH (off for a common-cathode LED). |  |
| setLED() takes an LED index (0-7), a colour (R,G,B) and a boolean (true = on). It writes LOW to the proper MCP pin, which sinks current through the resistor and lights the LED. |  |
| loop() steps through LEDs 0-7, turning the red leg on for 200 ms, then off; then the green leg; then the blue leg. The result is a “running-light” that lets you verify each of the 24 pins is correctly wired. |  |

If you wired the LEDs as common-anode (i.e., the anode to +3.3 V and you source current), just flip the logic inside setLED() – write HIGH for on and LOW for off.

## 5️⃣ Safety & current-budget checklist

| Item | Recommendation |
| --- | --- |
| Per-pin current | Keep each colour ≤ 15 mA. With a 330 Ω resistor and a 3.3 V supply you’ll be in the 5-10 mA range – comfortably inside the MCP’s 25 mA absolute rating. |
| Total chip current | The MCP23017 can source/sink ≈ 70 mA total (the sum of all pins). In this demo only one colour bank (8 pins) is active at a time, so the worst-case for a chip is ~ 8 × 10 mA ≈ 80 mA – a little above the spec. To stay strictly inside the spec, either: • Reduce the LED current a bit more (use 470 Ω resistors → ~ 6 mA per LED), or • Never light all eight LEDs of the same colour simultaneously (the demo only lights one at a time). |
| Power supply | Use the ESP-32’s 3.3 V rail (max ≈ 500 mA). Even with all LEDs at full 15 mA you’d draw ~ 120 mA, well within the ESP-32 regulator’s capability. |
| I²C pull-ups | Most MCP23017 breakout boards already have 4.7 kΩ pull-ups to 3.3 V. If yours does not, add two external 4.7 kΩ pull-up resistors (one on SDA, one on SCL). |
| Decoupling | Place a 0.1 µF ceramic capacitor close to each MCP's VDD-GND pins (most breakout boards already include one). |

## 6️⃣ What to try next (when the basic test works)

- Light multiple LEDs at once – add a routine that turns on several colours simultaneously (watch the total current).

- Create colour mixing – turn on two of the three colour pins for a single LED to get secondary colours (e.g., red + green = yellow).

- Add a third MCP23017 (or use the unused Port B of the second chip) to control additional LEDs or other peripherals (buttons, relays, etc.).

- Move to PWM dimming – if you need smooth fades, replace the MCPs with a PWM driver such as the PCA9685 (16-channel 12-bit PWM) or TLC5940 (constant-current PWM).

- Integrate with your existing game – once you have setLED(led, colour, state) working, you can call it from the game-logic part of your original sketch (instead of toggling GPIO 27/13/14) to give a visual cue for the leading colour.

### TL;DR – The short answer

- Yes – two MCP23017 chips can safely drive 8 common-cathode RGB LEDs (24 pins) as long as you keep each colour under ~10 mA with a resistor (330 Ω is a good start).

- Wiring: I²C SDA = GPIO 21, SCL = GPIO 22, address pins set to 0x20 and 0x21, red on Port A of chip 1, green on Port B of chip 1, blue on Port A of chip 2, each output through a resistor to the LED colour pin, LED common cathode to GND, anodes to +3.3 V.

- Test sketch: the code above initialises the expanders, sets all pins as outputs, and cycles each colour of every LED one-by-one so you can see everything light up in sequence.

Upload the sketch, power the board, and you should see the LEDs “walk” through red → green → blue on each of the eight LEDs. If any LED doesn’t light, double-check the wiring (pin-to-resistor-to-LED-to-+3.3 V) and the I²C addresses.

Happy hacking! 🎨🚀