## RGB Invaders – “How to Play”

Below is a complete, player-focused rule-book that explains everything you need to know to pick up the console, fire the right colours, survive the waves and finish the campaign.
All information comes directly from the source code, so it reflects the actual behaviour of the firmware (no guesswork).

### 1. What the device is

| Item | Description (from the sketch) |
| --- | --- |
| Processor | ESP32-WROOM DevKit v1 |
| Display | A WS2812B/WS2812 “NeoPixel” strip (configurable, max 480 LEDs). The strip is used as the whole game screen. |
| Buttons | Four coloured “action” buttons + one “Continue” button: BLUE → GPIO 25 RED → GPIO 33 GREEN → GPIO 26 WHITE → GPIO 32 (also the reset button) CONTINUE → GPIO 27 |
| Power-limit | The firmware caps the strip to ≈ 2.5 A (≈ 5 V × 2.5 A). If you exceed it the strip will dim automatically. |
| Audio | Audio code has been stripped – the game plays no sound (calls are still logged to the serial monitor). |
| Web UI | A tiny web server runs on the ESP-32 (default SSID “ESP-RGB-INVADERS”, password “12345678”). It is only needed to change the configuration (LED count, colours, level data, Wi-Fi, etc.). The UI is not required to play the game. |

### 2. Game-flow overview

[INTRO] → [PLAYING] → (if enemies cleared) → [LEVEL-COMPLETED]

└─► (if a boss is present) → [BOSS-PLAYING] → (if boss cleared) → [LEVEL-COMPLETED]

When LEVEL-COMPLETED:

• “You Win” animation runs (≈ 4 s)

• The “sacrifice LED” (LED 0) starts blinking – this is the **Continue** hint

• You may press the **Continue button (GPIO-27)** to jump straight to the next level

• If you do nothing, the game automatically goes back to the intro of the next level after the animation

If the enemy line reaches the home-base (the first `config_homebase_size` LEDs) → **Base destroyed** → GAMEOVER.

### 3. Buttons and their actions

| Button | Short press (< 1 s) | Long press (> 3 s) | Remarks |
| --- | --- | --- | --- |
| BLUE (25) | Fires a blue shot (colour 1). | – |  |
| RED (33) | Fires a red shot (colour 2). | – |  |
| GREEN (26) | Fires a green shot (colour 3). | – |  |
| WHITE (32) | RESET – ends the current run and returns to a brand-new game (all enemies, boss pieces, scores are cleared). If you hold it > 3 s the board goes into Wi-Fi configuration mode (the ESP creates an AP, you can then reach the web UI). | – |  |
| CONTINUE (27) | Only works while the “You Win!” screen is active (the sacrifice LED is blinking). A short tap jumps to the next level without resetting the score. | – | If you press it at any other time it is ignored. |
| Combination (Boss 3 only) | When playing the RGB Overlord (Boss 3) you can fire combo shots by pressing multiple colour buttons at once (see section 5). | – |  |

Cooldown – After any shot you must wait ≈ 100 ms (FIRE_COOLDOWN) before you can fire again. The firmware also enforces a global “button-released-first” rule, so you can’t hold a button down to spam shots.

### 4. The playing field (LED strip)

| Area | Meaning |
| --- | --- |
| LED 0 (optional “sacrifice LED”) | A dedicated LED that is always set to a dim red (CRGB(20,0,0)). It is used as a visual cue for the continue mechanic. While a level-completion animation is waiting, it blinks on/off at 2 Hz. |
| Home-base (config_homebase_size LEDs, default 3) | The leftmost config_homebase_size LEDs are always white. If the enemy line ever reaches these LEDs the base is destroyed → GAMEOVER. |
| Enemy / Boss line | The rest of the strip (from the first “playable” LED up to the last LED) shows the moving enemy wave or boss segments. The front of the line (the right-most visible part) advances toward the home-base. |
| Shots | Your shots travel right → left (low index → high index). They are drawn in the colour you fired. If a shot reaches the enemy line it is evaluated for a hit. |
| Boss projectiles | When a boss fires, its projectiles travel left → right (high index → low index). If a projectile reaches the home-base the base is destroyed. |
| Colour key (fixed in the firmware) | 1 = Blue (col_c1) 2 = Red (col_c2) 3 = Green (col_c3) 4 = Yellow (mix 1) (col_c4) 5 = Magenta (mix 2) (col_c5) 6 = Cyan (mix 3) (col_c6) 7 = White (player combo shot) (col_cw) 0 = Black (off) |
| Boss colour codes | Boss 1 and Boss 2 use the enemy colours 1-3. Boss 3 uses mix colours 4-6 and also white (7) for some special projectiles. |

### 5. Shooting mechanics

#### 5.1 Normal enemies (Levels 1, 2, 4, 5, 7, 8, 9)

- One-button shots – Press the colour button you want to fire.

- Hit condition – The shot must match the colour of the first enemy in the line. If it does: the enemy disappears, you gain a kill and the line shifts one step forward.

- Mistake – If you fire the wrong colour, the shot adds a new enemy of that colour to the front of the line (pushing the whole wave one step closer to your base). This is the “penalty” you want to avoid.

#### 5.2 Boss 1 – Masterblaster (Level 3)

- The boss consists of three colour blocks that move together.

- It fires single-colour projectiles (random colour 1-3, sometimes matching the front colour) at a configurable rate (boss1Cfg.shotFreq).

- Your normal single-button shots work exactly as for normal enemies – you must match the colour of the front boss segment.

#### 5.3 Boss 2 – The Tank (Level 6)

- The Tank is a 9-LED “segmented” boss split into three sections.

- Phases –

  - MOVE – the whole tank slides toward you.

  - CHARGE – the current section (0, 1 or 2) locks a colour (random 1-3). All LEDs of that section turn that colour.

  - SHOOT – the tank fires 10 shots of the locked colour, one every ~150 ms.

- Your job – While the tank is moving, you can shoot the front segment (single-button shot). When the tank is in CHARGE or SHOOT you can still fire, but only the locked colour will be effective; any other colour is ignored (the tank is “shielded”).

#### 5.4 Boss 3 – RGB Overlord (Level 10)

- This boss uses the mix colours (4 = Yellow, 5 = Magenta, 6 = Cyan).

- Combo shots – You can fire a white (7) combo shot by pressing two or three colour buttons simultaneously (within a 60 ms buffer). The mapping is:

| Buttons pressed | Resulting shot colour |
| --- | --- |
| Blue + Red + Green | 7 (White) |
| Red + Green | 4 (Yellow) |
| Red + Blue | 5 (Magenta) |
| Green + Blue | 6 (Cyan) |
| single button | 1-3 (Blue/Red/Green) |

- How to fire a combo – Press the desired buttons together (they must be held for ≤ 60 ms). The firmware records the combination after the buffer expires and creates the appropriate shot.

- Boss behaviour – The Overlord moves toward you, fires single-colour projectiles sporadically, and occasionally enters a “phase-change / burst” mode where all its segments flash white and it launches a rapid burst of random coloured projectiles.

- Your job – The boss is vulnerable to any colour while it is moving. During a phase-change the boss is temporarily invulnerable (hits are ignored). Use your combo shots to match the mix colours that appear on the segments.

### 6. Scoring & Statistics

| What you get | How it is calculated |
| --- | --- |
| Kill points | entityCount × 100 × levelMultiplier (see source calculateLevelScore()). entityCount is the total number of “HP units” the level contained (enemies or boss HP). |
| Time bonus | If you finish faster than a target time, you receive a full time-bonus (equal to the base points). If you are slower, the bonus is scaled down linearly. Boss 3 gets a full bonus regardless of time. |
| Level-total | Base points + time bonus. This is added to the “currentScore”. |
| All-time high score and last three scores are stored in the ESP’s non-volatile memory (Preferences). They survive power-cycles. |  |
| Shot / Kill counters | The firmware tracks the total number of shots you have ever fired (stat_totalShots) and total enemy kills (stat_totalKills). They are shown on the web UI and saved at each level end. |

Important: The continue button keeps the currentScore – you don’t lose the points you earned on the just-finished level.

### 7. The “Continue” mechanic (new in V2)

- When it appears – After you defeat a level’s enemy wave or boss, the firmware enters STATE_LEVEL_COMPLETED.

- Visual cue – LED 0 (the sacrifice LED) blinks on/off every 250 ms.

- What you can do –

  - Tap the physical Continue button (GPIO 27). The game immediately starts the next level with the same score (continueToNextLevel()).

  - Do nothing – the animation finishes after ~4 seconds and the game automatically starts the next level anyway (same result, just a tiny delay).

- When it does NOT work – If you press Continue while a normal game is running, during a boss fight, or after a game-over, nothing happens.

Why this matters – The original version forced a full reset after every win; the added button lets you chain levels and keep a high score.

### 8. Reset / Home-position button (WHITE)

- Short press (≤ 3 s) → Full game reset (abortAndResetGame()).

  - All enemies, boss pieces, shots, timers, scores for the current run are cleared.

  - The game returns to the intro of the start level (config_start_level).

  - Your all-time high score and statistics are preserved (they are stored separately).

- Long press (> 3 s) → Enter Wi-Fi configuration mode (the ESP creates an AP named “ESP-RGB-INVADERS”).

  - The LED strip flashes blue every 250 ms while you hold the button.

  - Connect to that AP and browse to http://192.168.4.1/ to change Wi-Fi credentials, LED count, colours, level data, etc. (Not needed for gameplay).

### 9. Power / LED-strip safety hints (derived from the code)

- Maximum current is limited to 2.5 A. The firmware will dim the strip automatically if you try to drive more.

- In the UI the “Estimated current” (≈ (LEDs × 15 mA × brightness%)/1000) is displayed; stay below 5 A to avoid the external power supply being overloaded.

### 10. Quick-start cheat-sheet (for the player)

| Step | Action | Expected result |
| --- | --- | --- |
| 1 | Power on the board. The LEDs light up dimly and an intro bar for the current start level appears. | You see the “level bar” animation. |
| 2 | Press any colour button (Blue/Red/Green) – a shot appears at the left edge and moves right. | Shot colour matches the button pressed. |
| 3 | Match colours – When the front enemy (or boss segment) is, say, red, press the Red button. | Enemy disappears, score counter increases. |
| 4 | Avoid mismatches – If you fire the wrong colour, a new enemy of that colour is added to the front, pushing the line toward you. | The line moves one step closer; you must be careful! |
| 5 | Boss fights – When a boss appears, watch its colour pattern: • Masterblaster (Level 3) – single colour segments, random shots. • The Tank (Level 6) – three phases; watch the “locked colour” (the colour of the section that glows red on the strip). • RGB Overlord (Level 10) – mix colours; use combo shots (press two or three colour buttons simultaneously). |  |
| 6 | Winning a level – The enemy line disappears → “You Win” animation. LED 0 blinks. | You can now tap the Continue button (GPIO 27) to instantly jump to the next level, or just wait for the auto-advance. |
| 7 | Losing – If the enemy line reaches the white home-base, the strip flashes red and the game ends (Game Over). | Restart via the White button. |
| 8 | Resetting – Short-press the White button any time to start a fresh run. | All current-run data cleared, back to start-level intro. |
| 9 | Changing Wi-Fi / Settings – Long-press the White button (> 3 s) → ESP creates an AP → connect with a phone/computer and open a web browser to 192.168.4.1. | From the web UI you can change LED count, brightness, colours, level parameters, etc. (not required for playing). |

### 11. “Cheat” / “Pro-tips” (what the code tells us)

| Tip | Why it works |
| --- | --- |
| Shoot as fast as possible – the FIRE_COOLDOWN is only 100 ms, so you can fire up to 10 shots/s if you can press fast enough. |  |
| Avoid the sac-LED – If you ever see the first LED (red) turning on while you are playing, it means you are already in a win-waiting state. Press Continue now to keep the momentum. |  |
| Boss 2 (The Tank) special – When the tank is charging, the locked colour flashes (you can see it as the segment colour changing to a solid hue). Only fire that colour during the subsequent shoot phase for guaranteed hits. |  |
| Boss 3 combo timing – The 60 ms input buffer means you have a very short window to press the buttons together. Press them simultaneously rather than sequentially. |  |
| Score optimisation – Finish each level faster than the target time (displayed in the UI as “Est. Current”) to get the full time-bonus (double the base points). For Boss 3 you always receive the full bonus, so focus on precision rather than speed. |  |
| Power-budget – If you raise config_brightness_pct to 100 % on a long strip (> 300 LEDs) the board may dim automatically because of the 2.5 A limit. Lower the brightness a bit to keep the visual intensity high. |  |

### 12. Summary of the three game states you’ll see

| State | What you see | What you can do |
| --- | --- | --- |
| STATE_INTRO | A coloured bar (size = level number) slides in the centre of the strip. | Nothing; just watch. |
| STATE_PLAYING / STATE_BOSS_PLAYING | Enemies or boss segments crawl toward the white home-base, your shots travel right → left. | Press colour buttons (and combos for Boss 3) to fire. |
| STATE_LEVEL_COMPLETED | A brief “level-won” animation, LED 0 blinks. | Press Continue (GPIO 27) to skip the short wait, or let the game auto-advance. |
| STATE_BASE_DESTROYED | Red flash, home-base turning to dark. | Press White to reset. |
| STATE_GAMEOVER | Whole strip solid red. | Press White to start a new run. |

### 13. Final note

All the above behaviour is hard-coded in the sketch you provided. No hidden menus, no hidden lives, no hidden power-ups – the game is a pure skill-test: match colour, manage the timing of your shots, and survive the increasingly aggressive bosses.

Enjoy the neon-rain of RGB Invaders! 🎉